Deepesh Oli

Enterprise L3 Spine-Leaf Data Center Network Design, OSPF Redundancy & Troubleshooting Project

Netmask: 255.255.255.252

| Link | Subnet | Spine IP | Leaf Ip |
| --- | --- | --- | --- |
| Spine-1 <->Leaf-1 | 10.0.1.0/30 | 10.0.1.1 | 10.0.1.2 |
| Spine-1 <->Leaf-2 | 10.0.1.4/30 | 10.0.1.5 | 10.0.1.6 |
| Spine-1 <->Leaf-3 | 10.0.1.8/30 | 10.0.1.9 | 10.0.1.10 |
| Spine-2 <->Leaf-1 | 10.0.1.12/30 | 10.0.1.13 | 10.0.1.14 |
| Spine-2 <->Leaf-2 | 10.0.1.16/30 | 10.0.1.17 | 10.0.1.18 |
| Spine-2 <->Leaf-3 | 10.0.1.20/30 | 10.0.1.21 | 10.0.1.22 |

Netmask: 255.255.255.248

| Link | Subnet | Vlan ip | Access Devices |
| --- | --- | --- | --- |
| Leaf-1 <-> vlan 10 | 10.0.1.24/29 | 10.0.1.25 | .26, .27 |
| Leaf-2 <-> vlan 20 | 10.0.1.32/29 | 10.0.1.33 | .34,.35 |
| Leaf-3 <-> vlan 30 | 10.0.1.40/29 | 10.0.1.41 | .42,.43 |

Troubleshooting the connections

Issue #1: Laptop 5 could not ping Network 10.0.1.24/29

- Symptom: ping failure

- Investigation: ping to other network works but unreliable, mostly packet drops

- Root cause: Laptop 5 of the leaf node 3 was in the same vlan 10 and network as laptops in leaf 1.

- Fix: Changed vlan and network of laptop5

- Verification: pinging works on all networks.

Issue #2: [short title]

- Symptom: Network is slow, pinging is hit or miss among all the networks

- Expected: ping among all the subnets

- Investigation: ping from laptop 0 to spine-2 10.0.1.21, 50 percent packet loss

- Root cause: vlans and network overlap between two leaf nodes

- Fix: shutdown of g 1/0/5 interface of leaf-3 and changed the vlan and network of g1/0/5 interface

- Verification: ping to laptop 0 (10.0.1.26) works

What I learned/achieved from this spine/leaf architecture

Spine-leaf provides equal cost, equal hop count, a predictable and simple way for every traffic.

Scalability is huge factor because we can increase port count by increasing leaf and bandwidth by increasing spines.

Every path is leaf->spine->leaf, so latency is always the same and predictable.

All paths are equal cost, so ECMP uses every spine at once. Redundancy and bandwidth are the same thing.

It scales independently: add leaves for more ports, add spines for more bandwidth.

Losing a spine costs 1/N of capacity, not connectivity.

Spine-to-spine links are useless, because every spine already reaches every leaf directly.

Connecting leaf to leaf or spine to spine breaks equal cost, equal hop property, creates bottleneck, makes design completed, difficult to identify problem and also scalability is affected.

The design is mostly used in modern datacenter architecture providing all above benefits.

The ping between same vlan at the ends of leaf was not achieved, because arp request on the same subnet does not go beyond the default gateway.

Limitation:

Packet Tracer limitations

- No VXLAN / EVPN overlay support so true data center overlay was not achieved.

- System limitation: I am using windows 11 arm based processor, so I couldn’t use EVE-NG and GNS3g for vxlan/evpn capabilities

Configuration

IP Address configuration

| Switch 0 ip routing int range g1/0/1-3 no switchport int g1/0/1 ip add 10.0.1.1 255.255.255.252 int g1/0/2 ip add 10.0.1.5 255.255.255.252 int g1/0/3 ip add 10.0.1.9 255.255.255.252 | switch 1 ip routing int range g1/0/1-3 no switchport int g1/0/1 ip add 10.0.1.13 255.255.255.252 int g1/0/2 ip add 10.0.1.17 255.255.255.252 int g1/0/3 ip add 10.0.1.21 255.255.255.252 |
| --- | --- |
| Switch 2 ip routing int range g1/0/1-3 no switchport int g1/0/1 ip add 10.0.1.2 255.255.255.252 int g1/0/2 ip add 10.0.1.14 255.255.255.252 | Switch 3 ip routing int range g1/0/1-3 no switchport int g1/0/1 ip add 10.0.1.6 255.255.255.252 int g1/0/2 ip add 10.0.1.18 255.255.255.252 |
| Switch 4 ip routing int range g1/0/1-3 no switchport int g1/0/1 ip add 10.0.1.10 255.255.255.252 int g1/0/2 ip add 10.0.1.22 255.255.255.252 |  |

Ospf configuration

| switch 0 hostname Spine-1 int lo0 ip add 1.1.1.1 255.255.255.255 ex router ospf 1 int g1/0/1 ip ospf 1 area 0 int g1/0/2 ip ospf 1 area 0 int g1/0/3 ip ospf 1 area 0 do wr | switch 1 hostname Spine-2 int lo0 ip add 2.2.2.2 255.255.255.255 ex router ospf 1 int g1/0/1 ip ospf 1 area 0 int g1/0/2 ip ospf 1 area 0 int g1/0/3 ip ospf 1 area 0 do wr |
| --- | --- |
| Switch 2 hostname leaf-1 int lo0 ip add 3.3.3.3 255.255.255.255 ex router ospf 1 int g1/0/1 ip ospf 1 area 0 int g1/0/2 ip ospf 1 area 0 do wr | switch 3 hostname leaf-2 int lo0 ip add 4.4.4.4 255.255.255.255 ex router ospf 1 int g1/0/1 ip ospf 1 area 0 int g1/0/2 ip ospf 1 area 0 do wr |
| switch 4 hostname leaf-3 int lo0 ip add 5.5.5.5 255.255.255.255 ex router ospf 1 int g1/0/1 ip ospf 1 area 0 int g1/0/2 ip ospf 1 area 0 do wr |  |

Leaf vlan config

| Leaf-1 int range g1/0/3-4 switchport mode access switchport access vlan 10 vlan 10 name Science int vlan 10 ip add 10.0.1.25 255.255.255.248 no shut | Leaf-2 int range g1/0/3-4 switchport mode access switchport access vlan 20 vlan 20 name Finance int vlan 20 ip add 10.0.1.33 255.255.255.248 no shut | leaf-3 int range g1/0/3-4 switchport mode access switchport access vlan 30 vlan 30 name IT int vlan 30 ip add 10.0.1.41 255.255.255.248 no shut |
| --- | --- | --- |

Vlan config

| int vlan 10 ip ospf 1 area 0 ex router ospf 1 passive-interface vlan 10 do wr | int vlan 20 ip ospf 1 area 0 ex router ospf 1 passive-interface vlan 20 do wr | int vlan 30 ip ospf 1 area 0 ex router ospf 1 passive-interface vlan 30 do wr |
| --- | --- | --- |